How to install an OmniPCXOffice-CPU

1. Introduction

The aim of this short document is to give a practical and simple example of how to install a complete software including IA, VoIP and software licenses using LOLA and PM5 with Ethernet direct connections.

2. References

[1] Software download for ALIZE-CPU & - LoLa R300
Ref :3AC_91000_5034_FAAFA_01
3. Procedure 1 (Software on the Hard Disk)

Action on  PM5 are in green, actions on LOLA are in purple and actions on CPU are in blue.

· Make sure that the software to install is stored on the PC, either locally (on Hard disk) or on a CD-Rom. If the software is stored on the network then it is necessary to follow the explanations of Procedure 2 (see in bottom)

· Set the DIP switch 3 to OFF on the CPU board.

· Plug the CPU board (without starting it).

· Start LOLA.

· Choose a software to install : 

· if you do not have safeguard of a configuration (xxx.lol) :

· select the delivery that you want to install

· select the country that you want to install

· if delivery is 300, select the CTI licence  that you want to install 

· if delivery is 300 ,select the Main licence that you want to install

· select the service that you want to install (for example IA , VOIP)

· if you have safeguard of a configuration (xxx.lol) :

· with the menu File/Open, select the file xxx.lol

· if you want, modify the country or the licence or the service

· Click Next

· Choose the type installation according to what you want to do (for example for a simple install choose : installation)

· Click Next
· Connect the Ethernet crossed cable between CPU and PC (LAN sockets).  

· Click Yes (DIP Switch and Ethernet  crossed cable  is OK)

· LOLA is waiting for Requests.

· Start the CPU (switch on).

After a couple of minutes the connection is established  and the download starts ( it then takes about 5 min.). 

· In the principal windows one has the course of the various stages of remote loading

· When the traffic is finish then the message of end of download and disconnection appears.
·  Switch off the CPU and wait until the led stops blinking.

· Unplug the CPU and set the DIP switch 3 to ON.

· Put the CPU back in a cabinet and switch it on; the CPU starts twice.
If the CPU board is meant to be plug in a cabinet with a different hardware configuration, proceed as follows:

· Program a manual Cold Reset (menu System particularities/system reset) : that means if you switch the CPU off and on again it will restart with default configuration.

· Switch off the CPU and wait until the led stops blinking.

· Unplug the CPU and plug it in the customer’s system.

The system will start cold with Software licenses and Download items previously downloaded.

4. Procedure 2  (Software on the network)

Action on  PM5 are in green, actions on LOLA are in purple and actions on CPU are in blue.

· Make sure that the software to install is stored on the network.

· Set the DIP switch 3 to OFF on the CPU board.

· Plug the CPU board (without starting it).

· Start LOLA.

· Choose a software to install : 

· if you do not have safeguard of a configuration (xxx.lol) :

· select the delivery that you want to install (on the Network)

· select the country that you want to install 

· if delivery is 300, select the CTI licence  that you want to install 

· if delivery is 300 ,select the Main licence that you want to install

· select the service that you want to install (for example IA , VOIP)

· if you have safeguard of a configuration (xxx.lol) :

· with the menu File/Open, select the file xxx.lol

· if you want, modify the country or the licence or the service

· Click Next

· Choose the type installation according to what you want to do (for example for a simple install choose : installation)

· Click Next

· Click Yes (DIP Switch and Ethernet  crossed cable  is OK)

· Wait a few minutes that LOLA copy the delivery on the Hard Disk

· LOLA is waiting for Requests.

· Connect the Ethernet crossed cable between CPU and PC (LAN sockets).
· Start the CPU (switch on).

After a couple of minutes the connection is established  and the download starts ( it then takes about 5 min.). 

· In the principal windows one has the course of the various stages of remote loading

· When the traffic is finish then the message of end of download and disconnection appears.
·  Switch off the CPU and wait until the led stops blinking.

· Unplug the CPU and set the DIP switch 3 to ON.

· Put the CPU back in a cabinet and switch it on; the CPU starts twice.
If the CPU board is meant to be plug in a cabinet with a different hardware configuration, proceed as follows:

· Program a manual Cold Reset (menu System particularities/system reset) : that means if you switch the CPU off and on again it will restart with default configuration.

· Switch off the CPU and wait until the led stops blinking.

· Unplug the CPU and plug it in the customer’s system.

The system will start cold with Software licenses and Download items previously downloaded.

END

